CHECKLIST FOR SOARING

2023-05-22

PREPARATION - NIGHT BEFORE

___ Car: Fuel, Oil, Coolant

___ Batteries : Glider, Handheld, GPS

___ Water bottles

___ Charts

___ PalmNAV & GPS loaded

___ Parachute ( repack date )

___ Oxygen

___ Granola Bars

___ Catheters

___ Landout Kit

___ European Link

___ Tiedown Kit

___ Sunburn Lotion

___ First Aid Kit

___ Book to read

___ Road Maps

___ Flashlight

___ Toolkit

___ Gap Seal Tape

___ Sunglasses

___ Hat

___ Cooler: drinks, lunch

ASSEMBLY

___  Main Pins Safetied

___  ELT Armed

___  Main Batteries Installed

___  LNAV Checked

___  Radio Checked

___  GPS Checked

___  Parachute In Cockpit

___  Relief System OK

___  Tire Pressure 35 psi

___  Landing Gear Doors

___  Tow Release Checked

___  Wing Roots Taped

LEFT WING

___  Winglet Installed/Taped

___  Zig-zag Tape OK

___  Mylar Seals OK

___  Aileron Drive OK

___  Rear Static Ports Clear

TAIL

___  Tail Weight Installed

___  Multi Probe Installed

___  Tail Bolt Installed

___  Tail Taped

___  Tail Wheel 35 psi

___  Mylar Seals OK

___  Zig-zag Tape OK

___  Rudder Cable Attachments OK

RIGHT WING

___  Aileron Drive OK

___  Zig-zag Tape OK

___  Mylar Seals OK

___  Winglet Installed/Taped

___  Wings Washed

___  Fuselage/Tail Washed

___  Canopy Clean

___  Water Ballast Filled/Valves OK

___  Oxygen Bottle Filled/Installed

GRID

___  Drinking Water Loaded

___  Food

___  Landout Jacket Stowed

___  Wallet Stowed

___  Survival Kit

___  Cell Phone

___  Map

___  Hat

___  Sunglasses

___  Prescription Glasses

___  Pen

___  Towel

___  Tie Down Stakes/Ropes

___  TOST Adapter Rings

___  Flashlight

___  Oxygen Mask Connected

___  Valve Open

___  Catheter On

TASK CHECK

___  Task Sheet

___  GPS Tasks In

___  Turnpoint Book

___  Turnpoint Matrix

___  Radio Frequencies Stored

___  Landing Card with Retrieve Phone Number

___  Map

IN COCKPIT

___  Tail Dolly Off

___  Radio On

___  LNAV/GPS Set Up

___  Wind Direction Checked

___  Air Vents Adjusted

___  POSITIVE CONTROL CHECK

PRE TAKEOFF

___  Controls

___  Ballast

___  Straps

___  Instruments

___  Flaps

___  Trim

___  Canopy

___  Brakes

___  Emergency

LANDOUT

___  LNAV Off

___  ELT Off

___  Wallet In Pocket

___  Landing Lat./Long.

___  Retrieve Number in Pocket

___  Cell Phone

___  Glider Secured

___  Multi Probe Stowed/Port Taped

___  Landing Card Signed

___  Retrieve Directions

TRAILER HOOK UP

___  Wingstands in Trailer

___  Hinged Top Latched and Safetied

___  Tie Down Ropes Removed

___  Front Door Latched Securely

___  Hitch Locked on 2"Ball/Checked

___  Dolly Wheel Raised and Locked

___  Safety Chain Connected

___  Break-away Wire Connected

___  Lights Connected and Checked

___  HAND BRAKE OFF ! ! !

___  Tires OK

___  Watch Out For Tie Down Stakes